# Energy and Hygrothermal Performance of Cross Laminated Timber Single Family Homes Subjected to Constant and Variable Electric Rates

Lewis Setter, Eric Smoorenburg, Sajith Wijesuriya, Paulo Tabares-Velasco*

Department of Mechanical Engineering

Colorado School of Mines, Golden, USA

# Abstract

Cross Laminated Timber (CLT) is a panelized mass timber product suitable for sustainable building construction that can reduce building energy use, and potentially reduce cooling electricity peak demand due to the additional thermal mass. The additional thermal mass of the CLT could be used to store cooling energy with an appropriate pre-cooling strategy and shift the electricity use during the peak time. The peak electricity demand during the summer driven by the need for cooling exerts a great stress on the US grid. The residential sector applications of CLT can relieve the stress on the grid. However, most of the current CLT literature has focused on tall buildings which are commercial in nature and therefore, the current study focuses on the residential buildings sector. Residential building energy use is highly susceptible to weather, thus they can potentially benefit from using CLT as the envelope material given its natural massiveness and enhanced air-tightness. This study numerically analyzes potential energy savings when CLT is implemented in a single-family home using BEopt, a residential building energy optimization program. A typical new light frame stud construction home is compared against the same house implemented with CLT construction. The effects of increased thermal mass and reduced infiltration are isolated to resolve the impact of CLT on energy consumption. This study also considers the use of different thermostat setpoints to introduce precooling strategies with the objective of reducing cooling electricity energy cost when subjected to time-of-use electricity rates. In addition, this study examines temperature and humidity changes in CLT homes using WUFI software platform, a wall assembly simulator used to analyze thermal and moisture changes. The thermal changes throughout the wall assemblies indicate the insulation performance of CLT, as well as ensure that the CLT assembly is physically practical, meaning that there is no moisture or humidity accumulation that can lead to rot or mold.

# 1. Introduction

Mass timber construction has recently seen worldwide interest and increased use due to its multiple benefits such as structural robustness and high level of pre-fabrication[1]. Timber buildings have been shown to have low global warming potential (GWP) compared to mineral based buildings, with cross laminated timber (CLT) capable of obtaining the lowest GWP of timber structures [X, new 2].   CLT is a panelized timber product that has established itself as a viable and environmentally friendly building technology in Europe and is seeing increased use in the rest of the world [2, 3]. It has widespread applications in medium rise to high rise buildings and provides numerous benefits such as modular construction, dimensional stability that decrease the overall construction time, and has superior fire performance to other building construction systems [4, 5] [6].  This provides an opportunity for CLT to be used in rapidly growing urban areas where ease of construction is must. Unlike in Europe, CLT is not yet widely used in the United States despite current literature paving the way for the implementation of CLT. Most of the research being done with CLT is targeted at medium and high rise buildings. Pei et al, for instance, conducted a study on the path to implementing CLT in tall buildings in the US and found that the economically viable market is between 8~20 stories [7]. Even though research on CLT in medium and low-rise building applications has increased in the US, research on the use of CLT in single family homes buildings lacks attention. Previous studies have evaluated the overall construction cost of building a single family home using CLT and have estimated the construction cost of CLT single family homes to be about 20% higher than comparable stud frame homes [8]. However, such studies have failed to numerically analyze energy use and/or hygrothermal performance in the feasibility of CLT constructions.

The previous results on high-rise CLT buildings suggest that single family homes could also make use from many of the benefits that CLT offers [9]. One of the benefits of working with CLT is reduced air leakage due to improved construction methods potentially making single family homes more energy efficient than light wood stud frames constructions by providing tighter control of airflow in and out of a building. Given the susceptibility of single family homes to weather the additional thermal mass of CLT panels can help regulating the indoor climate by storing sensible heat and creates an opportunity to control built environment with reduced heating and cooling energy [10]. Generally, homes in the U.S. have low thermal mass and therefore exhibit low storage potential [11, 12]. Similar to the use of phase change materials [13], a combination of thermal mass characteristics and programmable thermostats which cool the house during off-peak can be implemented to store cooling energy and reduce the cooling electricity requirement during the peak. Studies with phase change materials achieve energy cost savings up to 25% [14, 15] [16]. CLT can provide the additional thermal mass and air tighten requirements to provide effective precooling. Since PCMs store thermal energy in the form of both the latent heat and sensible heat, the storage capacity of CLT is expected to be lower than PCMs since CLT is only able to store thermal energy in the form of sensible heat.

Although CLT has the potential to reduce energy costs for a single-family home, CLT panels in a single-family home will be more susceptible to the environment than CLT panels in a mid-rise building. When designing and constructing mass timber buildings, engineers and architects have to consider the long term hygrothermal performance of CLT because its performance structurally, thermally, and in terms of fire resistance are all closely related to moisture content and relative humidity. Previous research has been found that, compared to light-weight timber construction, CLT offers less air permeability and has a greater capacity for storing humidity [17]. High relative humidity leads to the risk of mold growth on wood based construction products, including CLT. Along with the risk of mold growth is the risk of losing strength and stiffness of the Panel. Moisture content caused by high relative humidifies, (above 65%), have been shown to potentially reduce the stiffness in CLT [18]. It is therefore important to understand the wet and dry process and the long-term moisture content in key components of these multilayer mass timber buildings to assess their durability in different climates. Additionally, it is necessary to quantify the hygrothermal performance of single family homes constructed with CLT and compare the results to the performance of identical houses constructed using a typical wood stud frame designs in order to determine whether the modeled homes in this study are feasible from a hygrothermal standpoint in both a constant setpoint and variable setpoint scenario.

As a result, the objective of this study is to investigate the role thermal mass and reduced infiltration has on the thermal performance CLT homes compared to typical wood stud frame construction in three different climate zones in the United States. The contributions of this study are: (1) quantification of the benefits of thermal mass and reduced infiltration has on building energy use, (2) energy cost analysis for a combined strategy that includes precooling with a variable set-point schedule, CLT, and time-of-use residential electric plans and (3) hygrothermal analysis of CLT assembly when subjected to a constant and variable setpoints to determine the long term feasibility of CLT in a single family home provided the conditions of the energy performance models.

# 2. Methodology

The study compares the performance of a CLT home compare to a wood stud frame home in Sacramento, Phoenix and Boston. It combines building energy and hygrothermal software:

- Building Energy Optimization (BEopt) version 2.5 to create models of single family homes. BEopt performs annual building energy simulations using EnergyPlus as the simulation engine and can also perform parametric or optimization analyses using a sequential search optimization technique to find cost-effective energy packages [20] (Christensen, Anderson et al. 2006). EnergyPlus is a whole building energy simulation program, which uses an integrated, simultaneous solution approach to solve for different heat transfers processed in and out of the built environment

- WUFI version 6.X to perform ygrothermal simulation of the exterior walls, roof and …

These two software and the overall methodology followed is described below and in Figure U

- Energy analysis with constant setpoint. This step compares energy use for a CLT home with a reference that represents a typical single family home based on NREL House Simulation Protocols [19]. This step also analyzes the role infiltration has on CLT constructions by simulating a CLT home with its infiltration raised to the same level as a typical home.

- Energy analysis with variable setpoint. The step analyzes the ability of CLT homes to reduce energy cost when variable electricity rates are available. The reference home and the CLT home with a reduced infiltration are simulated with a variable cooling setpoint to precool during off-peak period.

- The third step analyzes and assess the impact precooling has on the hygrothermal performance of CLT over a five-year period under the conditions generated during the steps 1 and 2. The humidity and temperature in the CLT wall, floor, and attic assemblies are analyzed over a three-year period.

Figure 1. Data flow between different software and final analysis

The data flow is as follows.

- BEopt generates models of the reference home, the low infiltration CLT home, and the high infiltration CLT home for both constant and variable cooling setpoint schedules.

- EnergyPlus uses the models from BEopt along with TMY3 weather files as inputs and produces hourly energy data for a one-year period as an output.

- A Python script uses the variable setpoint E+ simulation data to calculate annual energy use with time-of-use price schedules.

- WUFI Pro uses the CLT construction assembly properties, the EnergyPlus predicted indoor air temperature and relative humidity, and WUFI/ASHRAE weather data inputs to produce hourly hygrothermal data for a five-year period as an output.

- Final post-processing of hygrothermal data is done in MATLAB and Excel.

## 2.1. Building Description

This study uses Building Energy Optimization (BEopt) version 2.5 to create models of single family homes. BEopt performs annual building energy simulations using EnergyPlus as the simulation engine and can also perform parametric or optimization analyses using a sequential search optimization technique to find cost-effective energy packages [20] (Christensen, Anderson et al. 2006). EnergyPlus is a whole building energy simulation program, which uses an integrated, simultaneous solution approach to solve for different heat transfers processed in and out of the built environment

All simulated homes are based on a reference building (Figure 1) is defined by NREL House Simulation Protocols that meet 2009 IECC code [21] The house has a conditioned area of 232 m2, a garage, slab-on-grade, and an unconditioned attic with the following characteristics:

- Cooling set-point of (76 °F)

- Heating set-point of (71 °F)

- Electric air conditioner rated at SEER 13

- Natural gas heater rated at 78% AFUE

Natural ventilation allowed 7 days a week throughout the year

Figure SEQ Figure \* ARABIC : A rendering of the BEopt model of the single family home.

Table 1 compares the envelope systems of each simulated CLT homes. CLT envelopes are modified versions of the reference home. CLT thermal properties used in EnergyPlus are: thermal conductivity of 0.12 W/m K (0.83 Btu in/h ft2 F), a specific heat of 1590 J/kg C (0.38 Btu/lb-F), and a density of 560 kg/m3 (35 lb/ft3) [8].

Table 1: Comparison between the reference home and the CLT home [18].

| Parameter | Reference Home | CLT Home with High Infiltration | CLT Home with Low Infiltration |
| --- | --- | --- | --- |
| Exterior wall structural material | 90x40 mm (2x4 in) studs, with fiberglass insulation | 105 mm (4⅛ in) 3 ply CLT | 105 mm (4⅛ in) 3 ply CLT |
| Second floor structural material | 240x40 mm (2x10 in) studs | 178 mm (7 in) 5 ply CLT | 178 mm (7 in) 5 ply CLT |
| Roof structural material | 240x40 mm (2x10 in) studs | 105 mm (4⅛ in) 3 ply CLT | 105 mm (4⅛ in) 3 ply CLT |
| Exterior sheating | OSB | Xx cm of XPS insulation | Xx cm of XPS insulation |
| Infiltration | 7 @ ACH50 | 7 ACH50 | 2 ACH50 |

Table 3 compares the thermal storage potential (kWh/K) of the reference CLT home. Reference homes only considers the drywall was potential thermal storage while CLT homes considers drywall and CLTs. CLT can potentially provide 2.5-4 times more thermal storage than traditional wood-stud walls.

Table 3: Summary of surface area and sensible thermal mass for various components in the living zone.

|  |  | Thermal Mass (kWh / K) |  |  |
| --- | --- | --- | --- | --- |
|  | Surface Area (m2) | Reference Home | 3Ply CLT Home | 5Ply CLT Home |
| Partition Walls | 232 | 1.1 | 7.2 | 12.2 |
| Interzonal Walls/Floors | 53 | 0.14 | 1.6 | 2.8 |
| Exterior Walls | 182 | 0.71 | 5.6 | 9.6 |
| Ceilings | 232 | 0.55 | 7.2 | 12.2 |
| Floors | 213 | 5.9 | 6.6 | 11.2 |
| Furniture | 93 | 3.1 | 2.9 | 4.9 |
| Total | ~ 1000 | ~ 12 | ~31 | ~53 |

Hygrothermal simulations

For many organic based construction materials, such as wood, the humidity levels must be lower than 70% to prevent mold growth and rot. In addition, the humidity levels should not drop too low either, as then vapor permeability may be reduced and the board may experience warping. However, most lumber used in CLT panel assemblies are processed in accordance with ANSI/APA PRG 320, restricting relative humidity level below 65% at 68˚F. [22]

The hygrothermal analysis on the CLT homes focus on wall, attic, and second level floor, (the assemblies that contained CLT) having the same thickness in all three regions. All simulations started on January 1st, the simulations for the walls were ran for five years, and the simulations for the floor and attic were ran for three years.

Figure 2 shows the CLT wall assembly, from left (exterior) to right (interior): the exterior is made of composite wood panels, followed by an air gap, exterior polystyrene insulation, a weather resistive barrier (WRB), the 3-ply CLT panel and interior gypsum board.

Figure 3 shows the CLT floor assembly from left to right, which is equivalent to top to bottom, respectively: laminate flooring, underlay, an oriented strand board (OSB), the 5-ply CLT panel, and finally a gypsum board for the ceiling. The laminate flooring is the floor of the second level of the house and the gypsum board is the ceiling of the first level of the house.

Figure 4 shows the CLT attic assembly, from left to right, which is equivalent to top to bottom, respectively: with R5 cellulose insulation, 5-ply CLT panel, followed by a gypsum board for the bottom/ceiling.  All hygrothermal properties come from the WUFI database.

The orientation of the analyzed wall assembly is based on the direction from which the most predominant rain comes from, to test the most critical surface enduring the most wind and rain in the respective regions. The analyzed wall orientations are as follows: Phoenix East; Sacramento South-East; and Boston North-East. In addition, the initial relative humidity of the wall assembly was set to 80%, and a 1% water infiltration from rainfall was added just behind the WRB. This was done to test the CLT panel in a non-ideal scenario and see if it was still capable of drying throughout the years even if the WRB was breached. The interior climate data used in the WUFI hygrothermal analysis simulations came from the BEopt hose model with low infiltration (2 ACH50). It should be noted then that a house with good ventilation and materials with properly finished surfaces are at less of a risk of mold growth [Y, new 22].

Figure 2: CLT Wall Assembly

Figure 3: CLT Floor Assembly

Figure 4: CLT Attic Assembly

Electric rates

For the constant setpoint analysis, Utility costs are calculated using a constant rate for both gas and electricity use. The average electricity and gas rates from each simulation location are used to determine utility rates for the models Table 2 [22]. Gas rates are applied to heating energy and water heating energy only.

Table 2. Utility rates for every location for the constant setpoint simulations.

| City | Electric Rate $/kW-hr | Gas Rate $/therm |
| --- | --- | --- |
| Phoenix | 0.1218 | 1.6560 |
| Boston | 0.1981 | 1.4062 |
| Sacramento | 0.1702 | 1.1059 |

Table 3 provides information of the thermal mass of all the envelope components (There is no comparison between the different envelope materials though)

Table 3: Summary of surface area and sensible thermal mass for various components in the living zone.

|  |  | Thermal Mass (kWh / K) |  |  |
| --- | --- | --- | --- | --- |
|  | Surface Area (m2) | Reference Home | 3Ply CLT Home | 5Ply CLT Home |
| Partition Walls | 232 | 1.1 | 7.2 | 12.21 |
| Interzonal Walls/Floors | 53 | 0.14 | 1.64 | 2.79 |
| Exterior Walls | 182 | 0.71 | 5.65 | 9.58 |
| Ceilings | 232 | 0.55 | 7.2 | 12.21 |
| Floors | 213 | 5.9 | 6.61 | 11.21 |
| Furniture | 93 | 3.1 | 2.89 | 4.90 |
| Total | ~ 1000 | ~ 12 | ~31 | ~53 |

## 2.2. Pre-cooling Strategies

To calculate cost for the variable setpoint simulations, the study uses time of use (TOU) plans provided by electrical utilities in each city. Gas rates remain the same as in the constant setpoint simulation Phoenix uses the E-21 Super Peak TOU price plan from the Salt River Project, Sacramento uses the Residential TOU Option 1 price plan from the Sacramento Municipal Utility District, and Boston uses NSTAR’s Boston Edison Co. Optional Residential TOU price plan. Table 4, Table 5, and Table 6 describe the price plans in detail.

Table 4: Annualized TOU rate for Phoenix, E-21 Super Peak TOU Rate

| Electric Rate (USD/kWh) | Months | Period |
| --- | --- | --- |
| 0.297 | May – Jun, Sep – Oct | 3 PM – 6 PM |
| 0.082 | May – Jun, Sep – Oct | 6 PM – 3 PM |
| 0.350 | Jul – Aug | 3 PM – 6 PM |
| 0.084 | Jul – Aug | 6 PM – 3 PM |
| 0.123 | Jan – Apr, Nov – Dec | 3 PM – 6 PM |
| 0.075 | Jan – Apr, Nov – Dec | 6 PM – 3 PM |

Table 5: Annualized TOU rate for Sacramento, Residential TOU Option 1

| Electric Rate (USD/kWh) | Months | Period |
| --- | --- | --- |
| 0.111 | Jan – May, Oct – Dec | 8 AM – 11 AM 7 PM – 10 PM |
| 0.103 | Jan – May, Oct – Dec | 11 AM – 7 PM 10 PM – 8 AM |
| 0.244 | Jun – Sep | 3 PM – 10 PM |
| 0.115 | Jun – Sep | 10 PM – 3 PM |

Table 6: Annualized TOU rate for Boston, Boston Edison Co. Optional Residential TOU

| Electric Rate (USD/kWh) | Months | Period |
| --- | --- | --- |
| 0.133 | Jan – May, Oct – Dec | 10 PM – 9 AM |
| 0.236 | Jan – May, Oct – Dec | 9 AM – 10 PM |
| 0.134 | Jun – Sep | 5 PM – 8 AM |
| 0.312 | Jun – Sep | 8 AM – 5 PM |

Suitable pre-cooling techniques for similar building models have been previously studied for Phoenix, Arizona using PCMs as the Thermal Energy Storage (TES) technique [Brandt, 2015]. Hence, the precooling setpoints used in this study are the same as in the previous study. In Phoenix and Sacramento, the cooling setpoint is reduced to 19 °C (67 °F) for 5 hours leading up to peak pricing and then raised to 27 °C (80 °F) for the entirety of peak pricing before being lowered back to typical value of 24 °C (76 °). In Boston, the cooling setpoint is raised from 24 °C (76 °F) to 27 °C (80 °F) from 9:00AM to 4:00PM. Figure 3 shows the setpoints used in each location.

To calculate cost for the variable setpoint simulations, the study uses time of use (TOU) plans provided by electrical utilities in each city. Gas rates remain the same as in the constant setpoint simulation Phoenix uses the E-21 Super Peak TOU price plan from the Salt River Project, Sacramento uses the Residential TOU Option 1 price plan from the Sacramento Municipal Utility District, and Boston uses NSTAR’s Boston Edison Co. Optional Residential TOU price plan. Table 4, Table 5, and Table 6 describe the price plans in detail.

Table 4: Annualized TOU rate for Phoenix, E-21 Super Peak TOU Rate

| Electric Rate (USD/kWh) | Months | Period |
| --- | --- | --- |
| 0.297 | May – Jun, Sep – Oct | 3 PM – 6 PM |
| 0.082 | May – Jun, Sep – Oct | 6 PM – 3 PM |
| 0.350 | Jul – Aug | 3 PM – 6 PM |
| 0.084 | Jul – Aug | 6 PM – 3 PM |
| 0.123 | Jan – Apr, Nov – Dec | 3 PM – 6 PM |
| 0.075 | Jan – Apr, Nov – Dec | 6 PM – 3 PM |

Table 5: Annualized TOU rate for Sacramento, Residential TOU Option 1

| Electric Rate (USD/kWh) | Months | Period |
| --- | --- | --- |
| 0.111 | Jan – May, Oct – Dec | 8 AM – 11 AM 7 PM – 10 PM |
| 0.103 | Jan – May, Oct – Dec | 11 AM – 7 PM 10 PM – 8 AM |
| 0.244 | Jun – Sep | 3 PM – 10 PM |
| 0.115 | Jun – Sep | 10 PM – 3 PM |

Table 6: Annualized TOU rate for Boston, Boston Edison Co. Optional Residential TOU

| Electric Rate (USD/kWh) | Months | Period |
| --- | --- | --- |
| 0.133 | Jan – May, Oct – Dec | 10 PM – 9 AM |
| 0.236 | Jan – May, Oct – Dec | 9 AM – 10 PM |
| 0.134 | Jun – Sep | 5 PM – 8 AM |
| 0.312 | Jun – Sep | 8 AM – 5 PM |

# 3. Results

## 3.1. Constant Setpoint Analysis

Table 7 displays the annual energy costs for the reference home and the CLT home with low infiltration (2 ACH50 infiltration) for each city. Table 7 also shows alongside the annual costs of heating energy and cooling energy. CLT reduces energy related cost from 10 to 18%.

Table 7: Annual total energy cost, heating energy cost, and cooling energy cost in all three analyzed cities.

|  | Reference Home Energy Cost (USD) | CLT, 2 ACH50 Home Energy Cost (USD) | Cost Reduction |
| --- | --- | --- | --- |
| Phoenix |  |  |  |
| Total | $1,980 | $1,770 | 11% |
| Heating | $290 | $180 | 36% |
| Cooling | $490 | $410 | 17% |
| Boston |  |  |  |
| Total | $3,400 | $2,780 | 18% |
| Heating | $1,540 | $990 | 36% |
| Cooling | $50 | $40 | 29% |
| Sacramento |  |  |  |
| Total | $2,140 | $1,900 | 10% |
| Heating | $570 | $420 | 26% |
| Cooling | $100 | $70 | 31% |

The results of the constant setpoint analysis suggest that CLT constructions reduce the annual energy cost for heating and cooling in both hot, warm, and cold climates. Furthermore, energy cost reduction in cold climates greatly exceeds cost reduction in other climates. CLT reduces the annual heating cost in Boston by 36%, equating to a savings of nearly 600 USD in the cooling category alone. In Phoenix, the hottest climate, CLT reduces the annual cost of heating by 17%. However, this only amounts to a savings of 110 USD on heating energy.

Figure 3 illustrates the effect of CLT on annual energy use reduction and reinforces the annual cost reductions from Table 7. In every climate, the use of CLT reduces annual heating energy substantially. It also shows that even though cooling energy is reduced in each location, the highest reductions from Table 7 (Boston and Sacramento) are irrelevant because of the low level of energy use.

The shows how the discrepancies in energy use from Figure 4 correlate can be attributed the reduced infiltration of the CLT single family homes. It plots the sensible heating rate during the coldest day of year in each location and the sensible cooling rate during the hottest day of the year over their respective 24-hour periods in each city.

The sensible heating rate plots suggest that the improved heating performance of the CLT homes is almost entirely attributable to reduced infiltration. In Boston, the coldest city, the CLT home with a high infiltration reduces heating energy by only 4% compared to the reference home, as opposed to 36% for the low infiltration CLT home. Given the high cost of CLT constructions, this result suggests that air tightening retrofit solutions might achieve similar levels of heating energy reduction, and for a fraction of the cost. Another study, for instance, found that simple, inexpensive modifications to the envelope of a single-family home reduced the energy consumption of a single-family home by half of what the simulated CLT constructions manage [23].

The plots of sensible cooling rate show that in all 3 locations, the cooling rate of the CLT house with 7 ACH50 infiltration is about half way between the cooling rate of the reference house and that of the CLT house with 2 ACH50 infiltration during the day. However, during the night, the cooling rates of the 2 CLT models are very similar. This indicates that despite energy savings generated during the day, the added thermal mass of the CLT incurs an energy penalty because extra cooling is need at night to remove heat gained during the day from the CLT and meet the cooling setpoint. Surprisingly, however, peak cooling rate reduction is the nearly the same as annual cooling energy reduction, between 15% and 20%, despite the energy penalty. In fact, CLT reduces the cooling rate by 15% to 20% throughout the day. This suggests that energy penalty is negligible. Table 8 shows that the relationships in the heating rate and cooling rate plots are true in all 3 cities and lists clarifying information regarding the conditions of each city over the days appearing in the plots.

Table 8: Reduction in Heating Energy on the coldest day of the year and reduction in peak electricity purchase rate during the hottest day of the year for all three cities.

| City | Reduction in Heating Energy over 24 Hours on Coldest Day |  | Minimum Outdoor Temperature on Coldest Day | Reduction in Peak Electric Use Rate on Hottest Day |  | Maximum Outdoor Temperature on Hottest Day |
| --- | --- | --- | --- | --- | --- | --- |
|  | CLT @ 7 ACH50 | CLT @ 2 ACH50 |  | CLT @ 7 ACH50 | CLT @ 2 ACH50 |  |
| Boston | 4% | 36% | -13 °C | 10% | 17% | 37 °C |
| Sacramento | 0% | 22% | 4 °C | 12% | 19% | 40 °C |
| Phoenix | 33% | 67% | 11 °C | 10% | 16% | 43 °C |

## 3.2. Precooling Analysis

Figure 9 shows the cost reduction between the reference homes and the CLT homes in Sacramento and in Boston are marginal. The variable setpoint reduced cost by XX% in Phoenix, reducing electricity costs from the reference case by 240 USD. The analysis of CLT with variable setpoint show that the use of CLT in single family homes improves the effectiveness of precooling in both Sacramento and Phoenix and improves the effectiveness of setback in Boston.

Here explain table 9

Table 9: Annual electricity cost in all three cities using a variable cooling setpoint

|  | Sacramento | Phoenix | Boston |
| --- | --- | --- | --- |
| Reference | $ 1,010 | $ 1,310 | $1,270 |
| CLT | $ 970 | $ 1,080 | $1,210 |
| Electricity Cost Reduction | 4% | 17% | 5% |
| Peak Cooling Reduction | 45% | 33% | 31% |

Despite the low energy cost savings in Sacramento and Boston, CLT reduces the peak cooling energy by 30% or more in all three cities. Figure X shows indoor air temperature, the interior surface temperature of the exterior walls, and the purchased electricity rate during the hottest day of the year for each analyzed building and location.

The variable setpoint simulations show that CLT homes cool down both faster and to a lower temperature than the reference homes, indicating the precooling potential of the CLT homes. This is due to the  air tightness of the CLT constructions. The simulations further show that the CLT walls are less prone to temperature swings than the wood stud frame walls and can decrease in temperature for a longer period. Because these properties, the CLT homes reduce the purchased electricity rate during peak hours. In Phoenix, the warmest location, electricity costs during peak hours are 10% lower in the CLT home than in the reference home. Another benefit that the CLT homes experience is that there is a significantly smaller spike in the electricity demand at the end of the setback period.

An optimized setpoint schedule will offer greater reductions in energy costs. However, the purpose of this part of the study is not to identify the maximum benefit from CLT in a precooling scenario. Rather, it is to establish the feasibility as a material to aid in the implementation of precooling and determine the necessity of future studies.

## 3.3. Hygrothermal Simulation

Figure 5 through figure 10 illustrate the CLT relative humidity in the Boston house over a three-year period. The red line graphs the relative humidity in the CLT panel closest to the interior of the house (interior side). The blue line graphs the relative humidity in the middle of the CLT panel (mid side). The green line graphs the relative humidity in the CLT panel closest to the exterior climate (exterior side). Although the simulation ran for 5 years, after the 3rd year there were negligible differences in relative humidity, thus this Figure XY only show first three years. The relative humidity of the CLT panels in the floor and attic displayed minimal differences between the non-precooling and the precooling system, as the exteriod side stay around 100% due to rain penetrating the WRF (main driver). In both cases, exterior side and mid panel have a RH higher than 70%, which….

.

a)							b)

Figure 5: CLT panel relative humidity of Boston  for a) base case with constant setpoint and a) variable setpoint precooling the home

The CLT panels in the walls are shown for both the non-precooling and precooling system for each region. For all graphs, the -graphs represents the relative humidity while the x-axis is the time in 6-month intervals, starting with January 1st.

The Figure YY to II  show the CLT relative humidity  for the homes located in Phoenix and Sacramento with constant setpoint (a) and variable setpoint (b).  IN all cases, the interior side of the CLT panel follows the indoor relative humidity as expected Additionally, the are no significant differences in the relative humidity on the interior side of the CLT panel between the variable and constant setpoints, indicating little change when precooling is used.

a)							b)

Figure 6: Relative humidity of Phoenix wall assembly CLT panel, base case and precool

a)							b)

Figure 7: Relative humidity of Sacramento wall assembly CLT panel, base case and precool

One of the first things that should be noted is that the interior side of the CLT panel is closer to the interior climate, it is therefore most affected by it. The relative humidity on the interior side of the CLT panel follows the relative humidity of the inside of the house very closely, essentially imitating it. It can be safely assumed that, under these conditions, the CLT panel’s relative humidity on its interior facing side will largely be affected by whatever the interior climate is set to. Additionally, the relative humidity on the interior side of the CLT panel in the precool house does not significantly differ from its non-precooling counterpart, indicating little change when precooling is used. The most valuable information that can be gathered from the relative humidity on the interior side CLT panel is how little it affects the middle of the CLT pane

The relative humidity in middle of the CLT panel, , is more heavily influenced by the relative humidity from the exterior side of the CLT panel due to the1% rainwater infiltration on the exterior side of the CLT. For both Boston and Sacramento, where the exterior side of the CLT panel fails to decrease below XY%, the relative humidity in the middle of the panel fluctuates between 70% and 80%.

It is evident that the integrity of the CLT panels is largely dependent on the relative humidity on its exterior side toward the middle of the panel. Recall that there was a, simulating a breach in the WRB. With this breach, regions with more frequent rain saturate the exterior side of the CLT panel, causing it to reach 100% relative humidity. This is the case for both Boston and Sacramento. The relative humidity of the CLT’s exterior side for these two regions never even decrease below 70% as required. This is the case for both the houses with precool and without precool systems.

The relative humidity in middle of the panel, which doubles as a gradient between the relative humidity on the exterior and interior side of the CLT panel, is more heavily influenced by the relative humidity from the exterior side of the CLT panel. For both Boston and Sacramento, where the exterior side of the CLT panel fails to decrease, the relative humidity in the middle of the panel fluctuates between 70% and 80%. Also, it should be noted that the relative humidity in the middle of the panel follows the exterior side relative humidity with a several month delay. For example, in the first half of the year in Sacramento, the exterior side of the CLT panel experiences a sharp increase in relative humidity, and several months following, the relative humidity in the middle of the panel will see an increase in humidity. As the exterior side relative humidity begins to decrease, so will the relative humidity at the middle of the panel, but only after some delay.

The CLT houses in Phoenix, a  dry region that does not frequently receive rain, take a much different course  As the relative humidity of the CLT panel’s decrease  significantly by the first year. By the second year the exterior and mid panel relative humidity have decreased to 50%, well below the 70% benchmark and into a safe zone, where mold is less likely to grow and cultivate, despite the breech in the WRB. The occasional spikes in the exterior side’s relative humidity is to due to the year’s largest rain event, but after this spike the relative humidity rapidly decreases. The rapid decrease in relative humidity of the CLT panel is likely due to several factors, both of them environmental..

Figure XY shows…. The major difference between the analysis of the wall and that of the floor and attic is that the simulation ran for the first three years, and the relative humidity of the floor and attic was initially set at 68% instead of 80%. Using WUFI 68% is a more rational percentage to analyze the floor and attic at due to the fact that the climates on both sides of the floor are interior climates, and so it cannot be expected to see the similar drying attitudes that are experienced in the wall. The same rational goes for the attic.

Figures 8-10 are a set of graphs that illustrate the course of relative humidity in the CLT panels of the floor (a) and attic (b) in each region.. The green line graphs the middle of the CLT panel for both the attic and the floor. For the attic, the red line graphs the attic side of the CLT panel and the blue line graphs the ceiling side of the CLT panel (the ceiling of the 2nd story). For the floor, the red line graphs the floor side of the CLT panel (the 2nd level floor), and the blue line graphs the 1st level ceiling side of the CLT panel (the ceiling of the 1st story).

a)							b)

Figure 8: Relative humidity of Boston attic CLT panel and floor CLT panel

a)							b)

Figure 9: Relative humidity of Phoenix attic CLT panel and floor CLT panel

a)							b)

Figure 10: Relative humidity of Sacramento attic CLT panel and floor CLT panel

It should first be noted that the ceiling side of the CLT panel for both the attic as well as the floor assemblies are very similar to one another. Similar to the interior side of the CLT panel in the wall assembly, the relative humidity at this point closely follows the interior relative humidity due to its proximity to the interior climate.

Regardless of climate, in the floor assembly, the floor side of the CLT panel has the highest humidity for the entirety of the simulation. Both the floor side and the middle of the CLT panel see reduction in relative humidity below sixty percent and below, but at a slow and gradual slope. This is likely due to the flooring, overlay, and OSB, which are creating a type of barrier which makes it harder for moisture to escape in that direction (via the floor).

Finally, the CLT panel in the attic dries in an expected fashion. The rate at which the middle of the panel and the attic side of the panel is apparently dependent on the exterior climate. For example, the CLT panel in Phoenix sees the greatest reduction in relative humidity while the one in Sacramento sees the least. The ceiling side of the CLT panel in the attic follows the interior climate for the same reasons that the ceiling side of the CLT panel.

# 4. Conclusion

This study analyzes The conclusion of this study are as follows:

Based on the simulated homes, preliminary results shows that the use of CLT in the construction of single family homes results in annual energy cost savings across all climates considered in this study, with higher savings in cold climates than in hot climates. For example, The  CLT home in Boston,  shows reduction in annual heating energy of 36%, and 600 USD, while the CLT home in Phoenix,  shows a reduction in annual cooling energy of 17% resulting in a savings of only $100. Regardless, neither of these savings are sufficient to offset the 20% increase in construction cost of a CLT single family home over a typical wood frame home at the current cost of CLT.  However,  the additional mass and increase airtightness warrants further simulations in areas of the US such as Colorado and Minnesota and with other strategies such as natural ventilation, night cooling and increase flow rate. Moreover, optimized CLT home designs should be considered by future studies to determine the most suitable environments for residential CLT buildings.

In cold climates, the aanalyzed homes show that reduction heating cost is predominantly due to the infiltration decrease. In hot climates, cooling energy reduction is due to both the reduced infiltration of the CLT construction and the thermal characteristics of CLT. This occurs because the hot climates considered in this study experience low temperatures at night and cool the CLT. The cooled CLT then aids in keeping a CLT home at a comfortable temperature during the day. However, the thermal energy gained by the CLT during day needs to be removed at night by the cooling system to adequately control indoor climate. The increased nighttime cooling load helps explain why energy cost savings in hot climates are insignificant.

Precooling analysis preliminary demonstrated that CLT constructed single family homes have the potential to benefit from precooling. The CLT homes achieved cocooling energy cost savings 30-45% lower than the wood frame home in Phoenix and Sacramento, the reduction was 45%. Despite the large reduction in cooling energy during peak pricing, the CLT homes did not experience a significant reduction in annual electricity costs. The annual electricity cost of the CLT home in Phoenix was only 17% lower than the corresponding wood stud home. Even though CLT shows promise for benefiting from precooling, it is unlikely that even an optimized setpoint schedule would create enough energy cost savings to offset construction costs at the current cost of CLT. Regardless, the potential of CLT as a viable construction material to implement precooling in single family homes warrants further studies on a case-specific basis.

The integrity of the CLT panels depends heavily on the state of the WRB and the climate that the CLT house is located in. The CLT fares well and quickly dries to safe levels, well below 70% relative humidity, when used to build a house in a climate that is dry and devoid of frequent rain, even if the WRB has been breached or if the CLT has been in some way been exposed to sources of water, such as rain water. When CLT is used in more humid climates and climates with more rainfall, then it is imperative that the WRB does not fail. Any exposure to a water source can saturate half of the CLT board, bringing its relative humidity levels to 100% and encourage the growth of mold and rot, placing the CLT’s structural integrity at risk. This study also discovered that the relative humidity in the interior of the house does not greatly affect the CLT panel, unless the relative humidity has a considerable range. A CLT house with a precooling system displayed hardly any differences compared to a CLT house without a precooling system.

# 5. References

1.	Mallo, M.F.L. and O.A. Espinoza, Outlook for cross-laminated timber in the United States. BioResources, 2014. 9(4): p. 7427-7443.

2.	Harris, R., Cross laminated timber. Wood composites. Cambridge: Woodhead Publishing, 2015: p. 141-67.

3.	Chen, Y., Comparison of environmental performance of a five-storey building built with cross-laminated timber and concrete. Sustainable Building Science Program, University of British Columbia-Department of Wood Science, Vancouver, BC, Canada, 2012.

4.	Karacabeyli, E. and B. Douglas, CLT: Handbook Cross-laminated Timber. 2013: FPInnovations.

5.	Ceccotti, A., New technologies for construction of medium-rise buildings in seismic regions: the XLAM case. Structural Engineering International, 2008. 18(2): p. 156-165.

6.	Buchanan, A.H., Fire performance of timber construction. Progress in structural engineering and materials, 2000. 2(3): p. 278-289.

7.	Pei, S., et al., Progress on the development of seismic resilient Tall CLT Buildings in the Pacific Northwest. 2014.

8.	Burback, B., Potential of cross laminated timber in single family residential construction. 2016: Colorado School of Mines.

9.	Štefko, J. and J. Bednár. Wood Based Building Envelope Regarding Air Tightness. in Key Engineering Materials. 2016. Trans Tech Publ.

10.	Arababadi, R. and K. Parrish, Modeling and Testing Multiple Precooling Strategies in Three Residential Building Types in the Phoenix Climate. ASHRAE Transactions, 2016. 122(2).

11.	Dexter, M.E., Energy conservation design guidelines: including mass and insulation in building walls. ASHRAE Journal, 1980. 22(3): p. 35-38.

12.	Turner, W., I. Walker, and J. Roux, Peak load reductions: electric load shifting with mechanical pre-cooling of residential buildings with low thermal mass. Energy, 2015. 82: p. 1057-1067.

13.	Konstantinidou, C.A., W. Lang, and A.M. Papadopoulos, Multiobjective optimization of a building envelope with the use of phase change materials (PCMs) in Mediterranean climates. International Journal of Energy Research, 2018.

14.	Kosny, J., N. Shukla, and A. Fallahi, Cost analysis of simple PCM-enhanced building envelopes in southern US climates. Report submitted to Building Technologies Program-Building America Project, US DOE, 2012.

15.	Thiele, A.M., et al., Annual energy analysis of concrete containing phase change materials for building envelopes. Energy Conversion and Management, 2015. 103: p. 374-386.

16.	Cole, W.J., et al., Reduced-order residential home modeling for model predictive control. Energy and Buildings, 2014. 74: p. 69-77.

17.	Brandner, R., Flatscher, G., Ringhofer, A. et al. Cross laminated timber (CLT): overview and development. European Journal of Wood and Wood Products, 2016. 74(3): p. 331-351.

18.	Gülzow, A., Richter, K. & Steiger R., Influence of wood moisture content on bending and shear stiffness of cross laminated timber panels. European Journal of Wood and Wood Products, 2011. 69(2): p. 193-197.

19.	Hendron, R. and C. Engebrecht, Building America house simulation protocols. 2010, Office of Energy Efficiency and Renewable Energy (EERE), Washington, DC (United States).

20.	Christensen, C., et al., BEopt™ Software for Building Energy Optimization: Features and Capabilities. National Renewable Energy Laboratory (NREL) Technical Report, 2006.

21.	Wilson, E., et al., 2014 Building America House Simulation Protocols. 2014, National Renewable Energy Laboratory (NREL), Golden, CO.

22.	Association, A.T.E.W., Standard for performance-rated cross laminated timber. ANSI/APA PRG, 2012. 320.

23.	Nabinger, S. and A. Persily, Impacts of airtightening retrofits on ventilation rates and energy consumption in a manufactured home. Energy and Buildings, 2011. 43(11): p. 3059-3067.

24.	Standard, A., Standard 62.1-2004,". Ventilation for acceptable indoor air quality, 2004.